ZONE-BASED SMART PARKING MANAGEMENT SYSTEM

# ABSTRACT

Traditional parking facilities largely rely on manual ticket issuance, human-operated barriers, and cash payment, which are slow, error-prone, and difficult to scale. This report presents the design of a Zone-Based Smart Parking Management System that focuses on two core capabilities: real-time, power-efficient slot occupancy monitoring, and cashless payment through dynamic VietQR codes. Each parking slot is equipped with a battery-powered wireless sensor node built around a QMC5883L three-axis magnetometer, which detects the presence of a vehicle by sensing the local distortion of the Earth's magnetic field caused by the vehicle's metal mass. Each node is controlled by a low-power microcontroller (ATmega328P) that remains in deep-sleep mode between periodic readings and communicates slot status to a central ESP32 gateway over an nRF24L01+ 2.4 GHz radio link, transmitting only when a change in occupancy is detected. This event-driven, low-duty-cycle design allows each sensor node to operate for extended periods on small batteries without requiring wired power or continuous network connectivity. The ESP32 gateway forwards slot-status updates to a Local Server, which maintains a real-time occupancy dashboard and, at the point of vehicle exit, calculates the parking fee and generates a dynamic VietQR code for immediate, cashless settlement through any participating bank application. By combining a magnetic-sensing and RF-based occupancy layer with a local, offline-first payment backend, the system offers an affordable and energy-efficient alternative to camera-based or wired sensor approaches, making it particularly suitable for small and medium-sized parking lots.

# CHAPTER 1. INTRODUCTION

## 1.1 Background and Motivation

The rapid growth of vehicle ownership in urban areas has placed increasing pressure on parking infrastructure, particularly on the ability of small and medium-sized lots to track slot availability and collect payment efficiently. Camera-based occupancy detection, while effective, requires continuous image processing, careful mounting to avoid obstruction by neighboring vehicles, and a comparatively high power budget — making it a poor fit for lots that want a low-cost, battery-friendly monitoring layer. Wired sensors, such as ultrasonic modules connected back to a central controller, avoid the processing overhead of vision-based systems but introduce significant installation cost, since cabling must be run to every monitored slot.

This project instead adopts a magnetic-sensing approach: a vehicle's chassis and engine block are large ferrous masses that measurably distort the ambient magnetic field directly above them. A small, low-power magnetometer embedded at or near each slot can detect this distortion directly, without requiring a camera, a line-of-sight sensor, or a wired power/data connection. Combined with a low-power microcontroller that spends the vast majority of its time in a deep-sleep state, and a short-range radio (nRF24L01+) that only transmits when the occupancy state actually changes, this approach allows each slot sensor to run for extended periods on small batteries. On the payment side, VietQR — Vietnam's interbank QR-code payment standard — allows any customer with a banking application to complete a transfer in seconds by scanning a dynamically generated code, removing the need for cash handling or a proprietary payment terminal.

## 1.2 Problem Statement

The central problem addressed by this project can be summarized as follows: how can a small parking facility monitor per-slot occupancy in real time using a wireless, battery-powered sensing layer that minimizes power consumption and installation cost, while integrating this data with a local server that automatically calculates parking fees and generates cashless VietQR payment codes, without relying on cameras, wired sensors, or continuous internet connectivity?

## 1.3 Objectives

The general objective of this project is to design, implement, and validate a working prototype of a zone-based parking-slot monitoring and payment system built around magnetic sensing, low-power wireless communication, and local VietQR payment processing. The specific objectives are:

- Design a battery-powered slot-sensor node combining a QMC5883L magnetometer with a low-power microcontroller (ATmega328P) and an nRF24L01+ radio module.

- Implement a duty-cycled firmware routine in which the node remains in deep sleep, wakes periodically to take a single magnetic-field reading, and transmits a status update only when the occupancy state changes.

- Calibrate each sensor node against its installation environment to reliably distinguish an occupied slot from an empty one under real ambient magnetic conditions.

- Develop an ESP32-based gateway that continuously listens for status packets from all sensor nodes and forwards updates to the Local Server.

- Build a Local Server component that maintains real-time slot-occupancy state, calculates parking fees, and generates dynamic VietQR codes for exit payment.

- Evaluate the power consumption of the sensor nodes to confirm the design meets the project's energy-efficiency goals.

## 1.4 Scope and Limitations

In scope: the design and firmware of the magnetic-sensor slot nodes, the RF communication protocol between nodes and the ESP32 gateway, the Local Server logic for occupancy tracking and VietQR fee calculation, demonstrated on a small-scale prototype covering 3 to 5 parking slots.

Out of scope: large-scale, multi-lot, or multi-level commercial deployment; vehicle entry/exit license-plate recognition and the lost-ticket recovery workflow, which are treated as a separate subsystem of the broader parking system; and long-term, real-world battery-life validation beyond laboratory-scale testing.

Known limitations: magnetic-field readings can be affected by nearby ferrous structures (rebar, metal beams), requiring per-slot calibration at installation time; the current design assumes a single shared RF channel serving 3 to 5 nodes rather than a larger, multi-channel deployment.